Nat

5

# National 5 Assignment
Acceleration on a slope: Guide Sheet A

Variation of angle of slope with acceleration.

Apparatus

Runway, light gate(s), interface and computer, mask (single or double), rule, protractor, or metre stick.

Instructions:

- Organise the light gates and interface to measure the acceleration of the vehicle as the vehicle passes through the light gate(s). This can be using one light gate and a double mask or two light gates and a single mask.

- Set the runway up at an angle.

- Measure the angle of the slope.

- Place the light gate so that the mask cuts the beam as the vehicle passes.

- Release the vehicle and measure the acceleration as it passes through the light gate(s).

- Change the angle of the slope, often books can be useful to achieve this.

- Present your results.

Risk Assessment

- Placing a buffer at the end of the ramp or something soft for the trolley to land in is suggested so not to damage equipment.

- Do an electrical safety check by observing all the wires of the interface and light gates.

- Make sure the vehicle cannot become a trip hazard or land on feet, toes etc.

- Be observant to those around you.

- Do not block exits with the apparatus.