# National 5 Assignment
Solar Cell- angle: Guide Sheet A

Nat

5

Investigating Solar Cells (Angle).

Apparatus (angle)

Solar cell unit, small motor unit (or other load), desk lamp (40 or 60 W tungsten lamp), digital multimeters (ammeter and/or voltmeter), 4 mm leads, metre rule, protractor, clamp stand

Instructions:

- Connect a solar cell to an electric motor.

- Connect a voltmeter across the solar cell also connect an ammeter in series in the circuit (not shown).

- The voltmeter and/ or ammeter can be used to get an idea about the output of the solar cell or the power can be determined.

- Shine a desk lamp onto the solar cell so that the motor turns, other loads can be used.

- Measure the angle between the lamp and the solar cell, and the corresponding current and voltage from the ammeter and voltmeter respectively.

- Adjust the angle between the lamp and solar cell, tilting the solar cell backwards and forwards ensuring you are clear about which angle you are measuring.

Risk Assessment

- Check all electrical wiring.

- Desk lamps with metal shades can get very hot. Take care when moving them.

- Be careful if considering other types of lamp such as halogen lamp and fluorescents because they can emit significant UV.