# ELECTRICITY SUMMARY NOTES

| Voltage (potential difference) is the energy transferred to each coulomb of charge. It is measured in volts (V). 1 V = 1 J per C Current is the charge transferred per second. It is measured in amps (A). 1A = 1 C per s |  |  |  | Mains Voltage: 230V, 50 Hz Factors that increase resistance Increase: Temperature Length of wire Thickness of wire |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A.C. - alternating current - changes direction and magnitude - from the mains |  |  | D.C. - direct current - one direction only - from a battery Same magnitude | Ohms Law Experiment Adjust resistance of variable resistor and take readings of V & I |  | Ohms Law Graph |
| If the polarity of D.C. trace is reverse (the connections are swapped) the trace goes below the line by the same amount |  |  |  | Resistance opposes the flow of current (Ω) |  | Gradient, m = V/ I = R |
| Series Rules Is = I1 = I2… Vs = V1 + V2… RT = R1 + R2… Adding R = RT | Parallel Rules Is = I1 + I2… Vs = V1 = V2… 1/RT = 1/R1 + 1/R2… |  |  | Connecting meters Ammeters -> series Voltmeters -> Parallel Ohmmeters -> no power supply |  |  |
| Transistors are electronic switches |  |  |  | The input voltage controls the switch. |  |  |
| npn transistor Input voltage must be 0.7V or above to switch on the output device. |  |  |  | n channel enhancement MOSFET Input voltage must be 2V or above to switch on the output device |  |  |
| Output Devices All the components below change electrical energy into another form of energy |  |  |  |  |  |  |
| Input devices All the components below change another form of energy into electrical energy |  |  |  |  |  |  |
| Other components |  |  |  | More about LEDs Series resistor protects LED from high current Current flows in opposite direction the arrow |  |  |
|  |  |  |  | To calculate R: - VR = VS – VLED - Use R = VR/ I |  |  |
|  |  | High Light sensor Voltage across transistor increases above 0.7V, transistor switches ON LED switches ON |  |  | Low Light sensor Voltage across transistor increases above 0.7V, transistor switches ON LED switches ON |  |
|  |  | Switch on Low Temperature Voltage across transistor increases above 0.7V, transistor switches ON LED switches ON |  |  | Switch on High Temperature Voltage across transistor increases above 0.7V, transistor switches ON LED switches ON |  |
| Voltage (potential) dividers These divide a supply voltage between 2 resistors |  |  |  | Power is the energy transferred per second It is measured in Watts (W). 1W = 1 J per s | Ew=QV Energy supplied to the charges Ew Q V = Charge voltage |  |
| A fuse melts to break the circuit if the current is too high. Power rating less than 720W – 3A fuse Power rating above 720W – 13A fuse Some appliances with a power rating less that 720W require a 13A fuse as they have a high current on switch on. |  |  |  |  |  |  |
| Power = current x voltage Power = current 2 x resistance Power = voltage 2 /resistance Greater current/voltage = greater power developed |  |  |  |  |  |  |
| Charges and Electric fields - Opposites ATTRACT - Like/Similar REPEL |  |  |  | A charged particle experiences a force in an electric field |  |  |
| Capacitors store charge on their plates. Capacitance, C, is measured in farads (F). Capacitors can be used with resistors in series as timing devices. The bigger the value of the resistance and the bigger the value of the capacitor, the longer it takes to charge to Vs |  |  |  |  |  |  |