# National 5 Assignment
LED: Guide A

Nat

5

Variation of current  for an LED

Equpment: Multimeters with settings for resistance, current and voltage, or ohmmeter, voltmeter, ammeter, variable resistor, LED, leads, power supply.

Instructions:

A

V

- Connect the circuit as shown in the diagram having chosen a suitable protective resistor.

- Set the variable resistor to give the lowest potential difference across the LED and record the readings on the voltmeter and ammeter.

- Alter the variable resistor to increase the potential difference.

- Record the new readings on the voltmeter and ammeter.

- Repeat steps three to four.

- Reverse the power supply connections and repeat steps two to six.

Risk Assessment

- Check all the wires and ensure that the wires are not frayed

- The resistor could get hot, so avoid contact with this.

- Switch off the equipment when it is not in use.

- Do not overload the LED

- a

Mrsphysics takes no responsibility for any health and safety. It is the responsibility of the teacher and student to risk assess any practical activity they complete!

Sept 2023